ECE411 MP4 CP1

MP4 Progress Report

Tingkai Liu, Zhi Cen

1	Design Checkpoint 0: RV32I ISA and basic pipelining

1.1	datapath

1.2	instruction analysis

| Opcode | IF | ID | EX | MEM | WB | Note |
| --- | --- | --- | --- | --- | --- | --- |
| IMM | 1 | 1 | 1 | 0 | 1 |  |
| REG | 1 | 1 | 1 | 0 | 1 |  |
| LUI | 1 | 1 | 0 | 0 | 1 | Place u_imm into register |
| MEM | 1 | 1 | 1 | 1 | 1/0 | Not need to write back for ST |
| AUIPC | 1 | 1 | 1 | 0 | 1 | Add something to PC and place the result into register. |
| BR | 1 | 1 | 1 | 0 | 0 | Conditional branch |
| JAL | 1 | 1 | 1 | 0 | 1 | Unconditional jump. |
| JALR | 1 | 1 | 1 | 0 | 1 | Unconditional jump. |

2	Checkpoint 1: RV32I ISA and basic pipelining

2.1 	functionality and work distribution

| Functionality | Notes | Work Distribution |
| --- | --- | --- |
| a 5-stage pipeline cpu which can handle RV32I Instructions | 1. no hazard detection 2. full RV32I Instructions except FENCE*, ECALL, EBREAK, and CSRR | Zhi: split mp2 cpu into 5 stages, unit test each stage and test the whole cpu with provided tb. Tingkai: design control rom, assign control signals to each stage and connect all stages in datapath. |

2.2	update datapath

2.3	update design

2.3.1	Control ROM

module control_rom(

input rv32i_opcode opcode,

input logic [2:0] funct3,

input logic [6:0] funct7,

output rv32i_ctrl_packet_t ctrl

);

2.3.2	Update the design of cpu_control

| Before | Now |
| --- | --- |
| get decode information from each buffer and output all control signals from cpu_control. | decode instruction by control_rom and save them in ctrl packet. In cpu_control, only deal with signals that can’t be determined at ID stage. |

2.4	testing and bug log

| Testing Type |  |
| --- | --- |
| Unit test | if_unit_test, id_unit_test, ex_unit_test, wb_unit_test |
| Using source code | mp4_cp1.s |

| Bug | Reason | Note |
| --- | --- | --- |
| PC didn't change at branch | forgot to set ALU for EX when BR |  |
| Invalid data read from D memory | forgot to mask the low 2 bits for mem_addr | I-cache access is always 4 byte allign so no need for that |
| When EX indicates halt, the WB for previous inst may not finish | The way for detecting halt may need to change |  |
| missing information for rvfimon | For rvfi monitor, we need to save additional information in packet | data_mem_rdata == mdrreg_out |
| rd_addr doesn’t match | all signals for rvfi monitors should be sent from wb stage | Only commit at wb.load_buffers && wb.inst != nop |
| pc_wdata doesn’t match at beq inst | pc_wdata should be pc+4 or alu_out |  |

3	Design Checkpoint 1: L1 caches, hazards and static branch prediction

3.1	arbiter design

3.1.1	datapath

3.1.2	state machine

3.1.3	state description

| State | Description | Next | Output |
| --- | --- | --- | --- |
| WAIT | Wait until a access request happens from either I-cache or D-cache | If I-cache requests, to I_MEM. If D-cache requests, to D_MEM. I-cache request is check first (Prioritized) | 0 |
| I_MEM | Access memory according to the signals from I-cache | If mem_resp, to I_DONE | Pass signals of I-cache to physical memory |
| I_DONE | Finish I-cache request, and check whether need to do D-cache request | If D-cache requests, to D_MEM, else to WAIT | inst_mem_resp = 1 |
| D_MEM | Access memory according to the signals from D-cache | If mem_resp, to D_DONE | Pass signals of D-cache to physical memory |
| I_DONE | Finish D-cache request, and check whether need to do I-cache request | If I-cache requests, to I_MEM, else to WAIT | data_mem_resp = 1 |

3.2	static branch predictor

3.2.1	datapath

3.2.2	pc update table

| Opcode | PC |
| --- | --- |
| BRANCH | PC <- PC + (br_en? b_imm : 4) |
| JAL | PC <- PC + j_imm |
| JALR | PC <- {(rs1_out + i_imm) [31:1], 1’b0} |
| Other opcodes | PC <- PC + 4 |

3.2.3	control logic

If EX.opcode == op_branch && br_en

ID.pc = nop

ID.ctrl = nop

IF.pcmux_out = EX.alu_out

If ID.opcode == op_jal

IF.pcmux_out = IF.pc_out + ID.j_mm

If ID.opcode == op_jalr

IF.pcmux_out = {(ID.rs1_out + ID.i_imm)[31:1], 1’b0}

Else

IF.pcmux_out = IF.pc_out + 4

3.3	forwarding

3.3.1	datapath

3.3.2	control

4	Checkpoint 2: L1 caches, hazards and static branch prediction

4.1 	functionality and work distribution

| Functionality | Notes | Work Distribution |
| --- | --- | --- |
| Hazard, static branch prediction | 1. data hazard: forwarding 2. control hazard: static-not-taken branch prediction | Tingkai |
| L1 cache, arbiter | 1. arbiter connected to I-Cache, D-cache and cacheline adaptor 2. no cache coherence | Zhi |

4.2	update datapath

4.3	update design

4.3.1	updates in L1 cache

| Before | Now |
| --- | --- |
| mp3 cache takes two cycles for cache hit | Now our cache takes one cycle for cache hit by changing read to combinational logic |

4.3.2	updates in arbiter

| State | Description | Next(Before) | Next(Now) | Output |
| --- | --- | --- | --- | --- |
| I_DONE | Finish I-cache request, and check whether need to do D-cache request | If D-cache requests, to D_MEM, else to WAIT | WAIT | inst_mem_resp = 1 |
| I_DONE | Finish D-cache request, and check whether need to do I-cache request | If I-cache requests, to I_MEM, else to WAIT | WAIT | data_mem_resp = 1 |

4.3.3	updates in forwarding

4.3.4	updates in control hazard

4.4	testing and bug log

| Testing Type |  |
| --- | --- |
| Unit test | cache_unit_test, arbiter_unit_test, cache_top_unit_test |
| Using source code | mp4_cp2.s, factorial.s |

| Bug | Reason | Note |
| --- | --- | --- |
| iteration limit reached | For one-cycle hit cache, inst_mem_read and inst_mem_resp are circular assigned. |  |
| cannot switch to inst/data_mem_state after inst/data_fin_state | TODO | TODO |
| stall at one instruction | When the inst at mem is invalid, forgot to change the condition for done and load_buffer is never asserted | also need to change if branch prediction is added! |
| write nonzero data to x0 | When write to x0, should write 0 anyway |  |
|  | For the forward input at MEM, the result is still possible to come from EX |  |
| Stalling didn't get the forwarded value | Forgot the forward setting (only) for lw |  |

5	Design Checkpoint 2: Advanced Design Options

5.1	Roadmap

| Functionality | Points | Notes | Work Distribution |
| --- | --- | --- | --- |
|  |  |  | Zhi |
|  |  |  | Tingkai |